MONTHLY PLAYGROUND INSPECTION REPORT

School: ______________________________________  Inspection Date: ________________________________

Inspected By: ___________________________________________________

INSTRUCTIONS

Playgrounds must be inspected on the first school day of each week.  Please fill out this inspection completely – use NA if necessary.  Attach any work orders for repairs to this report and note any work completed by the custodial staff in the Comments section.  Keep a copy on site and send this form to the Maintenance Department on a weekly basis.

| Ground Surfacing | Circle One | Comments |
| --- | --- | --- |
| There is at least 12 inches deep resilient ground surface (wood chips) under and at least 6 feet around all play equipment | Y / N / NA |  |
| The area around the fixed equipment and the fall zone is free from foreign objects and obstructions (protruding roots, rocks, drain pipes, etc.) | Y / N / NA |  |
| Concrete supports are covered with at least 12 inches of ground cover | Y / N / NA |  |
| The play chip area is free from collections of standing water | Y / N / NA |  |
| The field is free of holes and uneven surfacing | Y / N / NA |  |
| The blacktop and sidewalks are free of holes, cracks and raised surface over ¼ inch | Y / N / NA |  |
| The grounds are free from litter and glass | Y / N / NA |  |

MONTHLY PLAYGROUND INSPECTION REPORT (PG 2)

School: ______________________________________  Inspection Date: ________________________________

Inspected By: ___________________________________________________

| Play Structures | Circle One | Comments |
| --- | --- | --- |
| There are no sharp edges, broken parts, pinching actions or loose bolts | Y / N / NA |  |
| All openings are either less than 3 ½ inches or greater than 9 inches to prevent trapping a child’s head | Y / N / NA |  |
| Wood surfaces are in good repair; no splitting, rot, or excessively worn or warped areas | Y / N / NA |  |
| There are no protrusions longer than 1/8” that can catch clothing | Y / N / NA |  |
| All cables, hooks, chains and links are in good repair (no frayed cables, worn hooks, chains and links, etc) | Y / N / NA |  |
| Painted surfaces are smooth and free from chips or peeling paint | Y / N / NA |  |
| Surfacing Type: | Surfacing Depth: |  |
| Moveable backboards (basketball) | Y / N / NA |  |
| Bleachers: Broken or missing boards, fasteners. Bent, broken or cracked support structures, proper tracking of bleachers when extended. | Y / N / NA |  |

MONTHLY PLAYGROUND INSPECTION REPORT (PG 3)

School: ______________________________________  Inspection Date: ________________________________

Inspected By: ___________________________________________________

| Other Fixed Equipment | Circle One | Comments |
| --- | --- | --- |
| Soccer goals, ball walls and backstops are securely anchored in the ground and free of protursions and sharp points | Y / N / NA |  |
| Bike racks, picnic tables and benches are securely anchored and in good repair | Y / N / NA |  |
| Basketball nets are nylon and in good repair | Y / N / NA |  |
| Fence is secure and free from protrusions | Y / N / NA |  |

SKETCH LOCATION OF NEEDED REPAIRS

MONTHLY PLAYGROUND INSPECTION REPORT (PG 4)

School: ______________________________________  Inspection Date: ________________________________

Inspected By: ___________________________________________________

COMMENTS/REMARKS

| _____ Attach work orders for requested repairs Maintenance Department |  |  |  |
| --- | --- | --- | --- |
| Date received ____________________ |  |  |  |
| WO#s assigned |  | __________ |  |
|  |  | __________ |  |
|  |  | __________ |  |
|  |  | __________ |  |

_____  Send copy to Maintenance Department

_____  Keep copy for custodial records

____________________________________________

Inspector’s Signature